# MINERVA CryoCell P&ID — Default Views Documentation (v4)

Project: MINERVA CryoCell — SCK CEN (MYRRHA/MINERVA Phase 1) Consultant: Mott MacDonald, Bristol UK — MMD 411066 Standard: SCK CEN AD_01.16 · Status: S2 — FOR ACCEPTANCE · RESTRICTED

## 1. Purpose

The v4 P&ID set is layered. Each SVG contains 24 named layers (groups), and the drawing ships with five pre-defined default views — curated layer combinations for different audiences and tasks. The view definitions are embedded in each SVG’s <metadata> block and mirrored in the interactive viewer (js/pid-viewer.js → VIEWS), so the same presets are available whether you open the SVG directly or use the HTML viewer.

## 2. The 24 Layers (draw order, bottom → top)

Layers draw in list order — the first entry is the bottom of the stack. Tag layer 12_Tags and overlays are placed late so they render on top of pipes and equipment.

Grouped by function:

- Frame / title: 00_Frame, 01_TitleBlock

- Process pipes (by class): 02_Pipe_D, 02B_Pipe_E, 03_Pipe_A, 04_Pipe_B, 04C_Pipe_WATER, 05_Pipe_AIR

- Equipment: 06_Equipment, 07_Vessels, 08_Valves

- Overlays: 08B_Valves_HORIZONTAL_OVERLAY (hidden by default)

- Instruments / signals: 09_Instruments, 10_Signals, 11_Controls

- Annotation: 12_Tags, 13_LineNames, 14_TerminalPoints, 15_Dimensions

- Reference: 16_Legend, 16_Legend_INTERACTIVE (hidden), 17_Notes_TOGGLEABLE (hidden), plus revision/notes layers.

Three layers are hidden by default: 08B_Valves_HORIZONTAL_OVERLAY, 16_Legend_INTERACTIVE, 17_Notes_TOGGLEABLE.

## 3. The Five Default Views

| View | Audience / use case | Shows |
| --- | --- | --- |
| FULL | Complete reference / acceptance review | All visible layers (everything except the 3 hidden-by-default). |
| PROCESS | Process engineers — focus on fluid path | Frame, title, all process-pipe classes, equipment, vessels, valves, line names, terminal points, legend. Hides most instrument/signal detail. |
| CONTROL | C&I engineers — focus on instrumentation | Frame, title, equipment outlines, instruments, signals, controls, tags, legend. Pipes shown light for context. |
| MAIN | General overview / drawing issue | Frame, title, primary process pipes, equipment, vessels, valves, tags, line names, terminal points, legend. |
| PRINT_MONO | Black-and-white plotting / AD_01.16 mono check | All process content in monochrome with inline line names and weight/dash differentiation; colour suppressed. |

### View details

- FULL — vis = layer not in HIDDEN_BY_DEFAULT. The horizontal-valve overlay, interactive legend and toggleable notes remain off until explicitly enabled.

- PROCESS — emphasises 02*–05* pipe classes, 06/07/08 equipment & valves, 13_LineNames, 14_TerminalPoints. Instrument layers (09/10/11) are off.

- CONTROL — emphasises 09_Instruments, 10_Signals, 11_Controls, 12_Tags; process pipes kept visible but visually subordinate for context.

- MAIN — the “issue” view: primary headers + equipment + valves + tags + names.

- PRINT_MONO — drives the mono rendering path so a colour-free A3 plot still conveys every class via names, weights and dash patterns (see Colour & Legend Guide).

## 4. Using the Views

- In the SVG: the <metadata id="minerva-frontmatter"> block lists each view and its layer visibility map (machine-readable, inside a CDATA section so the YAML --- fences don’t break XML parsers).

- In the HTML viewer: select a preset from the Views control; the layer panel updates to reflect which layers each preset enables. You can then fine-tune individual layers on top of any preset.

See INTERACTIVE_VIEWER_MANUAL.md for step-by-step viewer instructions.