# Layer Structure Guide — 14-Layer Toggleable Hierarchy

Every v2 SVG uses a strict bottom-to-top stack of 14 Inkscape layers (inkscape:groupmode="layer"), so each can be toggled independently in Inkscape, Illustrator, or any CAD tool that honours SVG layer groups.

Layers are emitted in draw order: Layer 0 is drawn first (bottom), Layer 13 last (top).

| # | Layer label | Contents | Notes |
| --- | --- | --- | --- |
| 0 | Background | Sheet fill, outer/inner frame, title block, header strip, version stamp | Always on |
| 1 | Scope | Scope-boundary diamonds (TPXYYYY), terminal/handover points | Defines the “last-meter” interface |
| 2 | Structures | Cavity geometry, outer shell, vacuum vessel | Rendered light grey as background context |
| 3 | Equipment | Vessels, heat exchangers, cavities, couplers, tuners, tanks | Major equipment glyphs |
| 4 | Piping-40 K | 40 K thermal-shield circuit | Red (#e00000) |
| 5 | Piping-4.5 K | 4.5 K supply circuit | Blue (#0033cc) |
| 6 | Piping-2 K | 2 K return circuit | Cyan (#00a6bd) |
| 7 | Piping-Water | DI cooling-water / utility circuit | Green (#00a000) |
| 8 | Valves | All valve symbols + 60 K guard / infrastructure / instrument-air services | Manual, control, safety, relief, MV bellows |
| 9 | Instruments | ISA sensor bubbles (TT, PT, LT, FT, …) | Field-mounted discrete instruments |
| 10 | Control | DIS interlock block, tuner limit switches, Lemo connectors, actuators | Control / interlock logic |
| 11 | Signals | Signal lines + heat-load callouts | Pneumatic / electric signal layer |
| 12 | Tags | All instrument tags, valve tags, buffer & hand-over annotations | Text layer |
| 13 | Legend | Drawing legend (classes, symbols, scope keys) | Always on |

## Emphasis behaviour by sheet type

- Process / Cryogenic sheets (Sheet 1): piping layers 4-7 are drawn at full weight & colour; only inline process devices (FT/PT/LT) appear on the instrument layer. Control layer (DIS etc.) is empty.

- Instrumentation sheets (Sheet 2): piping layers 4-8 are drawn as a faded grey backdrop so the full sensor suite, control layer, and signals stand out.

## Style-version interaction

The two style versions modify rendering weights/colours within these layers (see Drawing Index §2) but do not change the layer structure — the 14-layer hierarchy is identical across both _STANDARD and _CONTROL-CENTRIC files.