# MINERVA CryoCell P&ID v3 — A3 Printing Guide

All v3 sheets are authored at true A3 landscape — 420 × 297 mm with a viewBox of 0 0 1587.273 1122.430 user units (≈ 3.779 units/mm). The PDFs are already A3; the SVGs declare width="420mm" height="297mm".

### Recommended output

| Use | File | Notes |
| --- | --- | --- |
| Colour review / plotting | *_STANDARD.pdf | Full colour, process emphasised |
| Control-room / signal review | *_CONTROL-CENTRIC.pdf | Process greyed back, signals emphasised |
| Black-and-white plotting | *_STANDARD_MONO.pdf / *_CONTROL-CENTRIC_MONO.pdf | Line-weight + dash differentiation only |

### Printer settings

- Paper: A3 (420 × 297 mm), landscape.

- Scaling: 100 % / Actual size — do not “fit to page” (it rescales line weights and defeats the calibrated 0.25–1.0 mm hierarchy).

- Margins: borderless or ≤ 6 mm; the sheet already carries a 6 mm frame margin.

- Colour: use the _MONO files for B/W printers rather than letting the driver desaturate — the mono files are designed to read without colour.

### Text legibility (verified minimums @ A3)

| Element | Target | v3 setting |
| --- | --- | --- |
| Main equipment / line tags | ≥ 2.5 mm | 2.6 mm |
| Instrument bubble text | 2.0 mm | ~1.85–2.0 mm |
| Line / note callouts | 2.2 mm | 2.2 mm |
| Legend text | 1.8 mm | 1.8 mm |

### Line-weight hierarchy (at 100 %)

- Primary cryo trunk 1.0 mm → branch 0.7 mm.

- Secondary (DI water) 0.5 mm.

- Out-of-scope services 0.35 mm (dashed).

- All instrument signals 0.25 mm (pneumatic dash+//, electric dotted, hydraulic dash-dot).

### Regenerating PDFs from SVG

cd pid_project
python3 -c "import glob,cairosvg; [cairosvg.svg2pdf(url=s, write_to=s[:-4]+'.pdf') for s in glob.glob('output_v3/*/*.svg')]"

cairosvg preserves the 420 × 297 mm page size (≈ 1190.55 × 841.89 pt).

### Tip — declutter before printing

Use a default view (see DEFAULT_VIEWS_GUIDE.md) to hide layers you do not need, and turn the 16_Legend_TOGGLEABLE layer on if the print is going to someone unfamiliar with the symbol set.