# P&ID SVG Inventory & Initial Analysis Report

Project: MINERVA — Flow Schemes and Instrumentation (P&ID reproduction to ISO / ISA 5.1) Generated: 2026-06-03 Source archives: - 02 - Flow schemes and Instrumentation.zip (≈ 0.97 MB) - 02 - Flow schemes and Instrumentation_Master.zip (≈ 72.6 MB)

## 1. Extraction Summary

Both archives were unzipped into organized directories:

/home/ubuntu/pid_project/
├── extracted/
│   ├── standard/        # contents of "02 - Flow schemes and Instrumentation.zip"
│   └── master/          # contents of "...Instrumentation_Master.zip"
├── svg_source/          # de-duplicated working copies of the 2 unique SVGs
├── previews/            # rendered PNG previews of each SVG
├── analyze_svg.py       # analysis script
└── svg_inventory.md     # this report

### Archive contents

| File | standard.zip | master.zip | Type |
| --- | --- | --- | --- |
| PFD-PID MINERVA QCELL-LB.svg | ✅ | ✅ | SVG (P&ID) |
| PFD-PID MINERVA RFCELL seen by ACR.svg | ✅ | ✅ | SVG (P&ID) |
| MANIFEST_02 - Flow schemes and Instrumentation.html | ✅ | ✅ | Manifest |
| PID Nomenclature MINERVA CryoCell (QCELL-LB).xlsx | ✅ | ✅ | Tag nomenclature |
| QCELL - Auxilliary lines (NA.CP).xlsx | — | ✅ | Auxiliary line list |
| 2024-03-07 - Brainstorming QM instrumentation and controls.pptx | — | ✅ | Reference |
| PFD-PID of RFCELL - MASTER.pptx | — | ✅ | Source/master deck |
| PID MINERVA CryoCell (QCELL-LB).pptx | — | ✅ | Source deck |
| QSYS (and RFCELL) instrumentation location for LB and LBI.pptx | — | ✅ | Reference (65 MB) |

Note: The two SVG files are byte-for-byte identical across both archives (verified by MD5). The _Master.zip additionally contains the editable PowerPoint sources and supporting Excel nomenclature/auxiliary-line lists, which will be valuable references for the standards-compliant reproduction.

### Unique SVG files found: 2

| # | File name | MD5 | Size |
| --- | --- | --- | --- |
| 1 | PFD-PID MINERVA QCELL-LB.svg | 788e30df…99e | 1.70 MB |
| 2 | PFD-PID MINERVA RFCELL seen by ACR.svg | d7fc4243…99b | 0.96 MB |

## 2. SVG #1 — PFD-PID MINERVA QCELL-LB.svg

The primary, most detailed P&ID — the full QCELL / CryoCell cryogenic flow scheme with complete instrumentation.

QCELL-LB preview

### Document properties

| Property | Value |
| --- | --- |
| Internal docname | PFD_MINERVA_QCELL-LB.svg |
| Authoring tool | Inkscape 1.4 (2024-10-11) |
| SVG version | 1.1 |
| viewBox | 0 0 1527.2727 1080 |
| Physical size | 420 mm × 297 mm (ISO A3, landscape) |
| File size | 1.70 MB |

### Structure & element counts

| Element | Count |
| --- | --- |
| Total elements | 3,463 |
| path | 1,029 |
| tspan | 663 |
| g (groups) | 586 |
| text | 459 |
| marker (arrow/line-end defs) | 336 |
| circle | 153 |
| ellipse | 118 (instrument bubbles) |
| rect | 75 |
| use | 24 |
| image | 3 (raster insets) |
| pattern / clipPath | 1 / 5 |

### Inkscape layers — 63 layers (rich semantic segmentation already present)

The drawing is heavily layered. Key layer groups relevant to the segmentation goal:

- Process / piping: Piping and series instrumentation (×3), Piping, Bottom line and common, 6 - Main process line return + buffer tank (V), 5 - Main process line inlet + buffer tank (L), 4 - Coupler B intercept, 3 - Coupler A intercept + common

- Items / components: Vacuum vessel (×3), 2 K tank, CPLR 1, CPLR 2, CPLR circuit, CPLR cuff and port metal, CPLR pressure limiter, Bottom connection, Spare heaters, Flowmeter for AL, Headers A B C D

- Instrumentation: Instrumentation, Over-instrumentalization, Instrumentation Pcav Observer, Instrumentation naming zones, EH TT, EHTT

- Sub-systems / zones: RFCELL 1, RFCELL 2, INVAC (×3), QVB, QINFRA, QM - DUT at FREIA, QM - PeMo, He guard (×2), PeMo-HEGU1, Instrument air, Utilities NA.PS, Users

- Annotation: Namings, Naming and conditions, Comments (×2), General comments, Heat loads (×2), Terminal points, Legend, Versions, Scope division, pn levels, electrical, TC-LB

### Text / labels / tags

- 566 text strings present (labels, tags, notes, legend).

- 203 unique ISA-style instrument tags. Tag-prefix histogram:

| Prefix | Meaning (ISA 5.1) | Count |
| --- | --- | --- |
| TT | Temperature Transmitter | 67 |
| HL | Heat Load annotation | 53 |
| EH | Electric Heater | 20 |
| LS | Level Switch | 17 |
| HV | Hand Valve | 12 |
| CV | Control Valve | 10 |
| PT | Pressure Transmitter | 9 |
| SV | Safety / Solenoid Valve | 6 |
| CF | (CF-flange callouts) | 6 |
| LT | Level Transmitter | 3 |
| FT | Flow Transmitter | 3 |
| PL | Pressure Limiter | 2 |
| AA | Analysis/Alarm | 2 |
| RV | Relief Valve | 1 |
| HX | Heat Exchanger | 1 |

- Sample component / equipment labels: CAV.1, CAV.2, CPLR.1, CPLR.2, CPLR.A, CPLR.B, HX550, V501, V502, TUN.A/B, RAD, CWT.A/B, K1, K2, MV1, MV2.

### Color palette — 67 unique hex colors (process-line color coding present)

| Color | Hex | Uses | Likely meaning |
| --- | --- | --- | --- |
| ⬛ Black | #000000 | 1144 | Outlines, generic structure, text |
| 🟦 Blue | #0000ff | 372 | Process line A (4.5 K; 3 bar) |
| 🟥 Red | #ff0000 | 333 | Process line D (40 K; 14 bar) |
| 🟩 Green | #00ff00 | 246 | Coupler water / utility lines |
| 🟦 Cyan | #00ffff | 194 | Process line B (3.5 K; 27 mbar) |
| 🫒 Olive | #808000 | 160 | He-guard header (292 K; 1.15 bar) |
| 🟪 Magenta | #ff00ff | 151 | Instrument air |
| ⬜ White | #ffffff | 145 | Fills |
| Dark grey | #1a1a1a | 131 | Text/strokes |
| Light grey | #f2f2f2 / #999999 / #808080 | 76 / 73 / 54 | Devices-under-vacuum, shading |
| 🟢 Dark green | #008000 | 43 | QINFRA scope lines |
| Navy | #000080 | 8 | — |
| Salmon | #ffaaaa / #d35f5f | 13 / 8 | RFCELL instrumentation bubbles |
| Amber | #ffcc00 / #f37f35 | 7 / 5 | Highlights / heat |

The legend in the drawing explicitly defines line classes A 4.5 K/3 bar, B 3.5 K/27 mbar, D 40 K/14 bar, E 60 K/13 bar, plus instrument-class fills (LB CRYO, RFCELL, LBI-specific, Protection) and a “vacuum barrier” dashed boundary — directly matching the requested segmentation axes.

## 3. SVG #2 — PFD-PID MINERVA RFCELL seen by ACR.svg

A focused P&ID of the RFCELL water-flow / coupler module (“as seen by ACR”), including the DI-water module for FREIA and the cavity/coupler detail.

RFCELL preview

### Document properties

| Property | Value |
| --- | --- |
| Internal docname | PID MINERVA RFCELL seen by ACR.svg |
| Reference | SCK CEN / 84836013 (v.1.3 — May 2025) |
| Authoring tool | Inkscape 1.4 |
| SVG version | 1.1 |
| viewBox | 0 0 1527.2727 1080 |
| Physical size | 420 mm × 297 mm (ISO A3, landscape) |
| File size | 0.96 MB |

### Structure & element counts

| Element | Count |
| --- | --- |
| Total elements | 1,139 |
| path | 386 |
| marker | 221 |
| tspan | 166 |
| g (groups) | 126 |
| text | 78 |
| circle | 57 |
| ellipse | 39 (instrument bubbles) |
| rect | 31 |
| use | 10 |
| image | 3 |
| pattern / clipPath | 2 / 9 |

### Inkscape layers — 25 layers

- Components: CPLR A, CPLR B, Tee A, Tee B, coldmasses, CAV, CPLR, CPLR water loops, CPLR flange heater, Tubes for FREIA, Green pipes

- Piping/process: RFCELL, RF tee anti-freeze, Green pipes

- Instrumentation: Instrumentation, EH TT (×2), CPLR AD, ED, PT, RF Tee monitoring (FREIA)

- Zones/annotation: background, TUN, Legend, Comments, Comment, Scopes, Version

### Text / labels / tags

- 128 text strings; 45 unique ISA-style tags. Prefix histogram:

| Prefix | Meaning (ISA 5.1) | Count |
| --- | --- | --- |
| TT | Temperature Transmitter | 23 |
| LS | Level Switch | 12 |
| EH | Electric Heater | 8 |
| PT | Pressure Transmitter | 6 |
| PZ | Pressure (special) | 4 |
| AD | Antenna/Analysis device | 4 |
| SM | — | 2 |
| RS | — | 2 |
| AP | Antenna Probe | 2 |
| SV | Safety Valve | 2 |
| ED | — | 2 |

- Note the x placeholder in tag names (e.g. TTx21, EHx11, LSx12, PZx21) — this drawing uses a generic/templated numbering scheme for the two symmetric cavity strings (A/B).

- Equipment labels: CAV.A, CAV.B, QCELL-QM, plus notes on the manual purging system.

### Color palette — 26 unique hex colors

| Color | Hex | Uses | Likely meaning |
| --- | --- | --- | --- |
| ⬛ Black | #000000 | 453 | Outlines, text |
| Navy | #000080 | 146 | Water-loop / process line |
| 🟦 Blue | #0000ff | 92 | Process line |
| Dark grey | #4d4d4d | 65 | Structure |
| ⬜ White | #ffffff | 64 | Fills |
| 🟥 Red | #ff0000 | 59 | Heater / hot line |
| 🟩 Green | #00ff00 | 48 | Coupler water lines (to FREIA) |
| Light grey | #f2f2f2 | 43 | Shading |
| 🟢 Dark green | #008000 | 34 | Green pipes / scope |
| 🟦 Cyan | #00ffff | 24 | Cold line |
| Brown | #aa4400 / #bf512e | 10 / 7 | Cavity/coupler bodies |
| Amber | #ffcc00 / #ff9a00 | 2 / 1 | Highlights |

## 4. Cross-Cutting Observations (relevant to the reproduction goal)

The requested segmentation axes map directly onto structures already present in the source SVGs, which makes a clean, standards-compliant rebuild feasible:

| Requested segmentation | Source signal already in the SVGs |
| --- | --- |
| Main lines | Color-coded line classes A/B/D/E + dedicated Piping…, Main process line… layers |
| Individual items | Equipment layers: Vacuum vessel, 2 K tank, CPLR 1/2, CAV, HX550, V501/V502 |
| Components | CPLR circuit, CPLR cuff and port metal, CPLR water loops, Bottom connection |
| Sensors | ellipse/circle instrument bubbles (118 / 39) + Instrumentation layers |
| Instrumentation tags | 203 + 45 unique ISA-style tags (TT, PT, LT, LS, FT, EH, CV, HV, SV, RV) |
| Vacuum barrier | Explicit dashed “vacuum barrier” boundary + INVAC layers |
| Temperature | TT/LT tags, EH heaters, Heat loads/HL annotations, cryo temperatures in legend |
| Pressure | PT/PZ/PL/RV tags, pressure values on each line class in legend |
| Color segmentation | 67 / 26 hex colors; line classes and instrument classes already color-keyed |

### Standards notes for the rebuild

- Both sheets are ISO A3 landscape with the same viewBox (0 0 1527.2727 1080), so a shared coordinate grid / template can be reused.

- Tags broadly follow ISA 5.1 letter conventions (first letter = measured variable, e.g. T, P, L, F; trailing function letters T = transmitter, V = valve, S = switch). A normalization pass against the PID Nomenclature MINERVA CryoCell (QCELL-LB).xlsx will be needed to make tags fully ISA-consistent.

- Instrument bubbles are drawn as ellipse/circle; line-end markers (336 / 221) are heavily used for arrowheads, heat-load triangles, and flange symbols — these will need a clean symbol library.

- Color usage is currently RGB-primary (#ff0000, #00ff00, #0000ff, …). For an ISO/ISA reproduction a defined, documented color palette should replace the ad-hoc primaries.

## 5. Inventory at a glance

| # | SVG | Sheet size | viewBox | Elements | Layers | Text | Unique tags | Colors |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | PFD-PID MINERVA QCELL-LB | A3 (420×297 mm) | 0 0 1527.27 1080 | 3,463 | 63 | 566 | 203 | 67 |
| 2 | PFD-PID MINERVA RFCELL seen by ACR | A3 (420×297 mm) | 0 0 1527.27 1080 | 1,139 | 25 | 128 | 45 | 26 |

Total: 2 unique P&ID SVG files, both Inkscape-authored ISO A3 landscape drawings sharing a common coordinate system, with existing semantic layering and color coding that align well with the planned ISO / ISA 5.1 segmentation and reproduction work.